# KPI Formulas Documentation

Complete Reference with Code Examples

# MILLING KPIs

## 1. Mill Throughput (%)

Formula: (cap_per_h / nameplate_tph) × 100

Scales Used: WG202, run_hours

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 231-241
nameplate_tph = 25.0

# Calculate capacity per hour
if cap_per_h <= 0 and run_hours > 0:
    cap_per_h = WG202 / run_hours if WG202 > 0 else 0.0

# Mill Throughput (%)
if cap_per_h > 0:
    mill_throughput = (cap_per_h / nameplate_tph * 100.0)
    # Cap at reasonable maximum (150% of nameplate)
    mill_throughput = min(mill_throughput, 150.0)

Example:

Input: WG202 = 200 tons, run_hours = 8 hours

Step 1: Validation - has_milling_data = True

Step 2: Calculate cap_per_h = 200 / 8 = 25 t/h

Step 3: mill_throughput = (25 / 25) × 100 = 100%

Step 4: Apply cap - min(100, 150) = 100%

Result: 100%

## 2. Mill Time Efficiency (%)

Formula: (run_hours / daily_hrs) × 100

Scales Used: run_hours, daily_hrs

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 244-247
if daily_hrs > 0:
    mill_time_eff = (run_hours / daily_hrs * 100.0)
    # Cap at 100% maximum
    mill_time_eff = min(mill_time_eff, 100.0)

Example:

Input: run_hours = 8 hours, daily_hrs = 24 hours

Step 1: Validation - has_milling_data = True

Step 2: mill_time_eff = (8 / 24) × 100 = 33.33%

Step 3: Apply cap - min(33.33, 100) = 33.33%

Result: 33.33%

## 3. Total Utilization (%)

Formula: (mill_time_eff × mill_throughput) / 100

Scales Used: mill_time_eff, mill_throughput (calculated)

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 250-252
total_util = (mill_time_eff * mill_throughput) / 100.0
# Cap at reasonable maximum
total_util = min(total_util, 150.0)

Example:

Input: mill_time_eff = 33.33%, mill_throughput = 100%

Step 1: total_util = (33.33 × 100) / 100 = 33.33%

Step 2: Apply cap - min(33.33, 150) = 33.33%

Result: 33.33%

## 4. Milling Gain (%)

Formula: ((WG501 + WG502 + WG503 + WG301 + WG302) / WG201) × 100

Scales Used: WG201, WG501, WG502, WG503, WG301, WG302

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 255-260
if WG201 > 0:
    total_output = WG501 + WG502 + WG503 + WG301 + WG302
    if total_output > 0:
        milling_gain = (total_output / WG201 * 100.0)
        # Cap at reasonable maximum (120%)
        milling_gain = min(milling_gain, 120.0)

Example:

Input: WG201 = 180t, WG501 = 120t, WG502 = 30t, WG503 = 48t, WG301 = 2t, WG302 = 1.5t

Step 1: total_output = 120 + 30 + 48 + 2 + 1.5 = 201.5 tons

Step 2: milling_gain = (201.5 / 180) × 100 = 111.94%

Step 3: Apply cap - min(111.94, 120) = 111.94%

Result: 111.94%

## 5. Milling Screening (%)

Formula: (WG301 / WG201) × 100

Scales Used: WG301, WG201

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 264-267
if WG201 > 0 and WG301 > 0:
    screening_ratio = (WG301 / WG201 * 100.0)
    # Cap at reasonable maximum (20%)
    screening_ratio = min(screening_ratio, 20.0)

Example:

Input: WG301 = 2 tons, WG201 = 180 tons

Step 1: screening_ratio = (2 / 180) × 100 = 1.11%

Step 2: Apply cap - min(1.11, 20) = 1.11%

Result: 1.11%

## 6. Flour Extraction (%)

Formula: ((WG501 + WG502) / WG202) × 100

Scales Used: WG501, WG502, WG202

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 272-277
if WG202 > 0 and (WG501 > 0 or WG502 > 0):
    total_flour = WG501 + WG502
    if total_flour > 0:
        flour_extraction = (total_flour / WG202 * 100.0)
        # Cap at reasonable maximum (85%)
        flour_extraction = min(flour_extraction, 85.0)

Example:

Input: WG501 = 120t, WG502 = 30t, WG202 = 200t

Step 1: total_flour = 120 + 30 = 150 tons

Step 2: flour_extraction = (150 / 200) × 100 = 75%

Step 3: Apply cap - min(75, 85) = 75%

Result: 75%

## 7. Milling Loss (%)

Formula: ((WG202 - (WG501 + WG502 + WG503)) / WG202) × 100

Scales Used: WG202, WG501, WG502, WG503

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 285-290
if WG202 > 0:
    total_output = WG501 + WG502 + WG503
    if total_output > 0:
        milling_loss = ((WG202 - total_output) / WG202 * 100.0)
        # Ensure it's not negative
        milling_loss = max(milling_loss, 0.0)

Example:

Input: WG202 = 200t, WG501 = 120t, WG502 = 30t, WG503 = 48t

Step 1: total_output = 120 + 30 + 48 = 198 tons

Step 2: milling_loss = ((200 - 198) / 200) × 100 = 1%

Step 3: Ensure non-negative - max(1, 0) = 1%

Result: 1%

## 8. Net Hours (hrs)

Formula: max(run_hours - downtime, 0.0)

Scales Used: run_hours, downtime

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Line 204
net_hours = max(run_hours - downtime, 0.0)

Example:

Input: run_hours = 8 hours, downtime = 0.5 hours

Step 1: net_hours = 8 - 0.5 = 7.5 hours

Step 2: Ensure non-negative - max(7.5, 0) = 7.5 hours

Result: 7.5 hours

## 9. Downtime (hrs)

Formula: Direct SCADA value

Scales Used: downtime

Validation Code:

# No validation - direct value from SCADA

Calculation Code:

# Line 133
downtime = safe(row.get("WG202_Stop_Start"), 0.0)

Example:

Input: downtime = 0.5

Result: 0.5 hours

## 10. Max Utilization of Milling Capacity (%)

Formula: (WG202 / (run_hours × 25.0)) × 100

Scales Used: WG202, run_hours

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 296-299
if run_hours > 0:
    max_utilization_milling_capacity = (WG202 / (run_hours * 25.0)) * 100.0
    # Cap at reasonable maximum (150%)
    max_utilization_milling_capacity = min(max_utilization_milling_capacity, 150.0)

Example:

Input: WG202 = 200 tons, run_hours = 8 hours

Step 1: max_utilization = (200 / (8 × 25)) × 100 = 100%

Step 2: Apply cap - min(100, 150) = 100%

Result: 100%

## 11. Pre Cleaning Screening (%)

Formula: (WG302 / WG101) × 100

Scales Used: WG302, WG101

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 303-306
if WG101 > 0 and WG302 > 0:
    pre_cleaning_screening = (WG302 / WG101) * 100.0
    # Cap at reasonable maximum (20%)
    pre_cleaning_screening = min(pre_cleaning_screening, 20.0)

Example:

Input: WG302 = 1.5 tons, WG101 = 185 tons

Step 1: pre_cleaning_screening = (1.5 / 185) × 100 = 0.81%

Step 2: Apply cap - min(0.81, 20) = 0.81%

Result: 0.81%

## 12. 1st Break Capacity per Hour (t/h)

Formula: WG202 / net_hours

Scales Used: WG202, net_hours

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 310-313
if net_hours > 0 and WG202 > 0:
    first_break_capacity_per_hour = WG202 / net_hours
    # Cap at reasonable maximum (30 t/h)
    first_break_capacity_per_hour = min(first_break_capacity_per_hour, 30.0)

Example:

Input: WG202 = 200 tons, net_hours = 7.5 hours

Step 1: first_break_capacity = 200 / 7.5 = 26.67 t/h

Step 2: Apply cap - min(26.67, 30) = 26.67 t/h

Result: 26.67 t/h

## 13. Bran Extraction (%)

Formula: (WG503 / WG202) × 100

Scales Used: WG503, WG202

Validation Code:

# Lines 229-230
if has_milling_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 317-320
if WG202 > 0 and WG503 > 0:
    bran_extraction = (WG503 / WG202) * 100.0
    # Cap at reasonable maximum (25%)
    bran_extraction = min(bran_extraction, 25.0)

Example:

Input: WG503 = 48 tons, WG202 = 200 tons

Step 1: bran_extraction = (48 / 200) × 100 = 24%

Step 2: Apply cap - min(24, 25) = 24%

Result: 24%

# PACKING KPIs

## 14. Packing Line Capacity (bags/hr)

Formula: PL601 / net_hours

Scales Used: PL601_TOT, net_hours

Validation Code:

# Lines 333-334
if has_packing_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 335-338
if net_hours > 0 and PL601 > 0:
    packing_capacity = (PL601 / net_hours)
    # Cap at reasonable maximum (2000 bags/hr)
    packing_capacity = min(packing_capacity, 2000.0)

Example:

Input: PL601 = 7500 bags, net_hours = 7.5 hours

Step 1: packing_capacity = 7500 / 7.5 = 1000 bags/hr

Step 2: Apply cap - min(1000, 2000) = 1000 bags/hr

Result: 1000 bags/hr

## 15. Daily Packing Output (bags)

Formula: PL601

Scales Used: PL601_TOT

Validation Code:

# Lines 333-334
if has_packing_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 341-346
if PL601 > 0:
    daily_packing_output = PL601
    # Cap at reasonable maximum (100000 bags per day)
    daily_packing_output = min(daily_packing_output, 100000.0)

Example:

Input: PL601 = 7500 bags

Step 1: daily_packing_output = 7500 bags

Step 2: Apply cap - min(7500, 100000) = 7500 bags

Result: 7500 bags

## 16. Net Hours (hrs)

Formula: max(run_hours - downtime, 0.0)

Scales Used: run_hours, downtime

Validation Code:

# Same as milling net hours

Calculation Code:

# Line 204
net_hours = max(run_hours - downtime, 0.0)

Example:

Input: run_hours = 8 hours, downtime = 0.5 hours

Result: 7.5 hours (same calculation as milling)

## 17. Downtime (hrs)

Formula: Direct SCADA value

Scales Used: downtime

Validation Code:

# No validation - direct value from SCADA

Calculation Code:

# Line 133
downtime = safe(row.get("WG202_Stop_Start"), 0.0)

Example:

Input: downtime = 0.5

Result: 0.5 hours

## 18. Machine Utilization (%)

Formula: (net_hours / daily_hrs) × 100

Scales Used: net_hours, daily_hrs

Validation Code:

# Lines 333-334
if has_packing_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 349-352
if daily_hrs > 0 and net_hours > 0:
    packing_util = (net_hours / daily_hrs * 100.0)
    # Cap at 100% maximum
    packing_util = min(packing_util, 100.0)

Example:

Input: net_hours = 7.5 hours, daily_hrs = 24 hours

Step 1: packing_util = (7.5 / 24) × 100 = 31.25%

Step 2: Apply cap - min(31.25, 100) = 31.25%

Result: 31.25%

## 19. Packing Line Capacity (tons/hr)

Formula: (PL601×0.045 + PL602×0.045 + PL603×0.040 + PL606×0.001 + PL607×0.010) / net_hours

Scales Used: PL601_TOT, PL602_TOT, PL603_TOT, PL606_TOT, PL607_TOT, net_hours

Validation Code:

# Lines 333-334
if has_packing_data and has_valid_data:
    # Only calculate if validation passes

Calculation Code:

# Lines 359-374
if net_hours > 0:
    # Get additional packing data
    PL606 = safe(row.get("PL606_TOT", 0.0))
    PL607 = safe(row.get("PL607_TOT", 0.0))
    
    # Convert bags to tons
    pl601_tons = PL601 * 0.045  # 45 KG bags
    pl602_tons = PL602 * 0.045  # 45 KG bags
    pl603_tons = PL603 * 0.040  # 40 KG bran bags
    pl606_tons = PL606 * 0.001  # 1 KG bags
    pl607_tons = PL607 * 0.010  # 10 KG bags
    
    total_packing_tons = pl601_tons + pl602_tons + pl603_tons + pl606_tons + pl607_tons
    packing_line_capacity_tons_per_hour = total_packing_tons / net_hours
    # Cap at reasonable maximum (50 t/h)
    packing_line_capacity_tons_per_hour = min(packing_line_capacity_tons_per_hour, 50.0)

Example:

Input: PL601=5000, PL602=2000, PL603=500, PL606=100, PL607=200, net_hours=7.5h

Step 1: Convert to tons

- PL601: 5000 × 0.045 = 225 tons

- PL602: 2000 × 0.045 = 90 tons

- PL603: 500 × 0.040 = 20 tons

- PL606: 100 × 0.001 = 0.1 tons

- PL607: 200 × 0.010 = 2 tons

Step 2: total_packing_tons = 225 + 90 + 20 + 0.1 + 2 = 337.1 tons

Step 3: capacity = 337.1 / 7.5 = 44.95 t/h

Step 4: Apply cap - min(44.95, 50) = 44.95 t/h

Result: 44.95 t/h

# WATER KPIs

## 20. Total Pre-Cleaning Water (m³)

Formula: DM101 + DM102 (incremental delta)

Scales Used: DM101, DM102

Validation Code:

# Lines 931-932
if current_scada_after_check and baseline:
    # Calculate delta (incremental)

Calculation Code:

# Lines 933-954
dm101_current = safe(current_scada_after_check.get("DM101", 0.0))
dm102_current = safe(current_scada_after_check.get("DM102", 0.0))

dm101_baseline = safe(baseline.get("DM101", 0.0))
dm102_baseline = safe(baseline.get("DM102", 0.0))

dm101_delta = max(0.0, dm101_current - dm101_baseline)
dm102_delta = max(0.0, dm102_current - dm102_baseline)

totalPreCleaningWater = dm101_delta + dm102_delta

Example:

Input (Current): DM101 = 50 m³, DM102 = 30 m³

Input (Baseline): DM101 = 35 m³, DM102 = 20 m³

Step 1: dm101_delta = 50 - 35 = 15 m³

Step 2: dm102_delta = 30 - 20 = 10 m³

Step 3: totalPreCleaningWater = 15 + 10 = 25 m³

Result: 25 m³ (incremental)

## 21. Water Clean Wheat (m³)

Formula: DM201 + DM202 + DM203 (incremental delta)

Scales Used: DM201, DM202, DM203

Validation Code:

# Lines 931-932
if current_scada_after_check and baseline:
    # Calculate delta (incremental)

Calculation Code:

# Lines 933-954
dm201_current = safe(current_scada_after_check.get("DM201", 0.0))
dm202_current = safe(current_scada_after_check.get("DM202", 0.0))
dm203_current = safe(current_scada_after_check.get("DM203", 0.0))

dm201_baseline = safe(baseline.get("DM201", 0.0))
dm202_baseline = safe(baseline.get("DM202", 0.0))
dm203_baseline = safe(baseline.get("DM203", 0.0))

dm201_delta = max(0.0, dm201_current - dm201_baseline)
dm202_delta = max(0.0, dm202_current - dm202_baseline)
dm203_delta = max(0.0, dm203_current - dm203_baseline)

waterCleanWheat = dm201_delta + dm202_delta + dm203_delta

Example:

Input (Current): DM201 = 40 m³, DM202 = 35 m³, DM203 = 25 m³

Input (Baseline): DM201 = 20 m³, DM202 = 20 m³, DM203 = 15 m³

Step 1: dm201_delta = 40 - 20 = 20 m³

Step 2: dm202_delta = 35 - 20 = 15 m³

Step 3: dm203_delta = 25 - 15 = 10 m³

Step 4: waterCleanWheat = 20 + 15 + 10 = 45 m³

Result: 45 m³ (incremental)

## 22. Total Water Used (m³)

Formula: DM101 + DM102 + DM201 + DM202 + DM203 (incremental delta)

Scales Used: DM101, DM102, DM201, DM202, DM203

Validation Code:

# Lines 931-932
if current_scada_after_check and baseline:
    # Calculate delta (incremental)

Calculation Code:

# Lines 933-954
# Calculate all deltas as shown in previous KPIs
dm101_delta = max(0.0, dm101_current - dm101_baseline)
dm102_delta = max(0.0, dm102_current - dm102_baseline)
dm201_delta = max(0.0, dm201_current - dm201_baseline)
dm202_delta = max(0.0, dm202_current - dm202_baseline)
dm203_delta = max(0.0, dm203_current - dm203_baseline)

totalWaterUsed = (dm101_delta + dm102_delta + 
                  dm201_delta + dm202_delta + dm203_delta)

Example:

Using deltas from previous examples:

- dm101_delta = 15 m³

- dm102_delta = 10 m³

- dm201_delta = 20 m³

- dm202_delta = 15 m³

- dm203_delta = 10 m³

Step 1: totalWaterUsed = 15 + 10 + 20 + 15 + 10 = 70 m³

Result: 70 m³ (incremental)